ROBIN ROBOT

Control App — User Manual

Complete guide to all 8 panels · Voice commands · Scan modes · Motion library

| Platform | Flutter Mobile / Desktop App |
| --- | --- |
| Robot IP | 10.255.254.75 (port 8000) |
| Wake Word | "z bot" — triggers all voice commands |
| Languages | English · Farsi · Arabic |
| Version | 1.0 ¦ Robin Robot System |

# 1. Overview

The Robin Robot Control App is a Flutter-based mobile and desktop application that connects to the Robin robot over Wi-Fi. It provides full control of all robot capabilities through 8 dedicated panels, accessible via a bottom navigation bar.

| # | Panel | What You Can Do |
| --- | --- | --- |
| 1 | 🤖 Robot Control | Start/shutdown robot, view live status, set face expressions |
| 2 | 🎤 Voice Commands | Activate wake-word listener, set language, view command log |
| 3 | 💬 AI Chatbot | Type or speak questions, robot answers via LLM |
| 4 | 🏃 Motion Library | Trigger 40+ pre-built motions across 6 categories |
| 5 | 🔍 Scan Mode | OCR text reading, ID/passport extraction, auto-describe |
| 6 | 📷 Camera | Turn camera on/off, capture still image |
| 7 | 👁 Vision | Start/stop object tracking and YOLO object detection |
| 8 | 🔊 Audio | Control ReSpeaker microphone, play audio files |

💡 TIP All panels communicate with the robot via HTTP API on port 8000. Ensure your device is on the same Wi-Fi network as the robot (10.255.254.75) before using the app.

# 2. Getting Connected

## 2.1 Connection Requirements

| Requirement | Detail |
| --- | --- |
| Wi-Fi network | Same network as robot (e.g. 10.255.254.x) |
| Robot IP | 10.255.254.75 |
| API Port | 8000 (HTTP REST) |
| Robot powered on | run.sh must be running on the robot |
| ROS2 running | robot_interface.launch.py must be active |

## 2.2 Startup Checklist

- Power on the Robin robot

- Wait ~45 seconds for all nodes to initialize

- Open the Robin app on your phone/tablet

- Navigate to Robot Control panel

- Tap "Get State" — if status data appears, you are connected

⚠ WARNING If Get State returns no data, the robot API server may not be running. SSH into the robot and run: bash /home/robin/robin_ws/run.sh

# 3. Panel 1 — Robot Control

The Robot Control panel is the main control hub. Use it to start or shutdown the robot, check live status, and control face expressions.

## 3.1 Power Control

| Button | Action |
| --- | --- |
| ▶ START ROBOT | Sends start command to robot. All nodes become active. |
| ⏹ SHUTDOWN | Shows confirmation dialog → gracefully stops all nodes. |

⚠ WARNING Always use the SHUTDOWN button to stop the robot. Cutting power abruptly may corrupt the face profile database.

## 3.2 Robot Status

The Status card shows live data from the robot refreshed every 15 seconds. Fields include:

- mode — current robot mode (idle, listening, tracking, etc.)

- face_expression — currently displayed face

- voice_active — whether wake word is listening

- ai_phase — current AI pipeline phase

## 3.3 Face Expressions

Tap any expression button to instantly change the robot's face display:

| Expression | Icon | When to Use |
| --- | --- | --- |
| HAPPY | 😄 | Welcome / positive feedback |
| SAD | 😢 | Error / negative feedback |
| ANGRY | 😠 | Warning / denied action |
| SURPRISED | 😮 | Unexpected event detected |
| NEUTRAL | 😐 | Default idle state |

💡 TIP You can also change expressions via voice: 'z bot, you look happy' will route through the LLM and may trigger an expression change.

# 4. Panel 2 — Voice Commands

This panel activates the robot's built-in wake word listener. Once active, the robot continuously listens for 'z bot' and responds to spoken commands.

## 4.1 Language Selection

| Code | Language | Notes |
| --- | --- | --- |
| en | English | Default. Best accuracy with Vosk. |
| fa | Farsi | Vosk model must be installed at /opt/models/vosk-fa |
| ar | Arabic | Vosk model must be installed at /opt/models/vosk-ar |

## 4.2 Controls

- BEGIN — activates wake word detection on the robot

- STOP — deactivates listening

- Language dropdown — sends language change to the robot before listening

## 4.3 All Voice Commands

Say 'z bot' followed by any command below. The wake word triggers a beep sound confirming detection.

### Movement Commands

| Voice Command | What Robin Does |
| --- | --- |
| "z bot, go forward" | Robot drives forward |
| "z bot, go back" | Robot reverses |
| "z bot, turn left" | Robot turns left |
| "z bot, turn right" | Robot turns right |
| "z bot, spin" | Robot performs a full spin |
| "z bot, stop" | Immediately halts all movement |
| "z bot, face me" | Robot turns to face the direction the voice came from (uses ReSpeaker direction) |
| "z bot, face back" | Robot turns 180° to face backwards |

### Gesture Commands

| Voice Command | What Robin Does |
| --- | --- |
| "z bot, wave" | Robot waves its arm/head |
| "z bot, dance" | Robot performs a dance sequence |
| "z bot, charge" | Robot performs charge motion |

### Vision Commands

| Voice Command | What Robin Does |
| --- | --- |
| "z bot, scan now" | One-shot auto-describe scan — robot describes what it sees |
| "z bot, read text" | OCR scan — robot reads all visible text aloud |
| "z bot, scan id" | ID scan — robot extracts Name, DOB, ID number from ID card |
| "z bot, describe scene" | Detailed 2-sentence scene description |
| "z bot, detect objects" | List all visible objects |
| "z bot, scan mode on" | Enable continuous auto-scan every 4 seconds |
| "z bot, scan mode off" | Disable continuous scanning |
| "z bot, what do you see" | Robot describes current view using YOLO detection |

### Face & Identity Commands

| Voice Command | What Robin Does |
| --- | --- |
| "z bot, checkface" | Robot checks face DB and announces who it sees |
| "z bot, identify" | Same as checkface |
| "z bot, follow me" | Robot tracks and follows a person using YOLO |
| "z bot, stop following" | Robot stops person-following mode |

### System Commands

| Voice Command | What Robin Does |
| --- | --- |
| "z bot, deactivate" | Robot enters sleep/shutdown mode |

💡 TIP The command log at the bottom of the Voice Commands panel shows timestamps and results of every command in real time.

# 5. Panel 3 — AI Chatbot

The Chatbot panel allows direct text or voice conversation with Robin's LLM (Zenon AIHIVE-LLMDSTXT). Responses are limited to 30 words for quick, robot-appropriate answers.

## 5.1 Text Chat

- Type any question in the text field

- Tap Send — the robot processes via LLM and replies in under 10 seconds

- All conversation appears in the scrollable terminal log

## 5.2 Voice Chat

- Select language from dropdown

- Tap BEGIN — robot starts listening for your question

- Speak naturally — no wake word required in chatbot mode

- Robot transcribes → sends to LLM → speaks reply via Piper TTS

- Tap STOP to end the session

## 5.3 Supported Languages

- English (en) — default

- Farsi (fa) — requires Vosk Farsi model

- Arabic (ar) — requires Vosk Arabic model

💡 TIP Response length is capped at 30 words. For longer explanations, ask follow-up questions.

# 6. Panel 4 — Motion Library

The Motion Library provides one-tap access to all pre-programmed robot movements, organized into 6 expandable categories. Tap any motion tile to execute it.

## 6.1 Categories

| Category | Example Motions | Notes |
| --- | --- | --- |
| 🚶 Basic | forward, backward, turn left/right, stop | Wheel-based movement commands |
| 🕺 Gestures | wave, dance, spin, charge | Pre-programmed joint sequences |
| 😊 Face | face_me, face_front, face_left, face_right | Head/torso orientation to sound direction |
| 🎭 Expressions | happy, sad, surprised, neutral | Triggers face_node expression change |
| 🔧 Utility | stand, reset, calibrate | Reset/home position commands |
| ⚡ Advanced | charge, complex sequences | Multi-joint choreographed sequences |

## 6.2 Running a Motion

- Tap the category header to expand/collapse it

- Tap any motion tile — the button shows the emoji and motion name

- A spinner appears in the header while the motion runs

- Status bar shows: Running → Completed → clears after 2 seconds

⚠ WARNING Only one motion can run at a time. Tapping another motion while one is running has no effect — wait for the current motion to complete.

# 7. Panel 5 — Scan Mode

Scan Mode uses the Zenon Vision AI (AIHIVE-LLMFILTXT) to analyze camera images. The robot can read text, extract ID card fields, or describe its surroundings.

## 7.1 Scan Modes

| Mode | Code | Description |
| --- | --- | --- |
| AUTO | auto | Continuous scan every 4 seconds — robot describes what it sees |
| OCR | ocr | One-shot: reads all text visible in frame (documents, signs, books) |
| ID | id | One-shot: extracts Name, ID Number, Date of Birth, Expiry from ID/passport |
| OFF | off | Disables continuous scanning |

## 7.2 Quick Scan Buttons

| Button | What It Does |
| --- | --- |
| Scan OCR | Immediately triggers a one-shot OCR scan, result spoken aloud |
| Scan ID | Immediately triggers ID extraction from whatever is in front of camera |
| Get Status | Fetches current scan mode status from robot |
| Get Result | Fetches the most recent scan result text |

## 7.3 Face-Aware Scanning

When a known person is in the camera view, Scan Mode automatically personalizes the description:

- Without known face: 'I see a person holding a paper...'

- With known face (Robin): 'I see Robin holding a paper...'

- Only live face recognition data is used — no stale database lookups

💡 TIP For best OCR results: ensure the document fills most of the camera frame and there is good ambient lighting. Hold the document still for 2-3 seconds before triggering the scan.

💡 TIP For ID scanning: place the ID flat, face-up, within 30cm of the camera. The robot will extract only clearly visible fields.

# 8. Panel 6 — Camera

The Camera panel controls the CSI ribbon-cable camera (IMX sensor) on the robot. The camera publishes to /camera/image/raw at 21Hz and is used by all vision features.

## 8.1 Controls

| Control | Action |
| --- | --- |
| Camera ON | Starts the camera_publisher_node and activates lifecycle |
| Camera OFF | Deactivates camera_publisher_node lifecycle |
| Capture Image | Saves a still frame from the camera stream |

⚠ WARNING Only one process can open the CSI camera at a time. The camera_publisher_node owns the hardware — do not run test scripts that open the camera directly while the robot stack is running.

💡 TIP The camera starts automatically when run.sh is executed. You do not normally need to use this panel unless the camera stopped unexpectedly.

# 9. Panel 7 — Vision

The Vision panel controls YOLO object detection and person-following mode. These features use the camera feed and are started on demand to save GPU memory.

## 9.1 Object Tracking (Follow Mode)

| Control | Action |
| --- | --- |
| START TRACKING | Starts object_tracker + follower_ctrl nodes via node_manager |
| STOP TRACKING | Deactivates both nodes, robot returns to stand position |

In follow mode, the robot uses YOLO to detect a person and continuously adjusts wheel speed to keep the person centered in frame. Parameters:

- Deadband: 0.07 — no movement if person is already centered

- Tracking speed: 10 RPM on each wheel

- Lost timeout: 400ms — stops if target not seen

## 9.2 Object Detection (YOLO)

| Control | Action |
| --- | --- |
| START DETECTION | Activates yolo_detector lifecycle node (YOLOv8 TensorRT) |
| STOP DETECTION | Deactivates yolo_detector |

- Detects up to 5 objects per frame at confidence > 0.5

- Publishes text descriptions to /yolo/detection_text

- Publishes JSON bounding box data to /yolo/detection_raw

💡 TIP You can hear what YOLO sees by saying 'z bot, what do you see' — the robot speaks the detection_text via TTS.

# 10. Panel 8 — Audio

The Audio panel manages the ReSpeaker 4-microphone array and allows playing audio files stored on the robot.

## 10.1 ReSpeaker Controls

| Control | Action |
| --- | --- |
| ReSpeaker ON | Activates microphone_array_publisher_node lifecycle |
| ReSpeaker OFF | Deactivates microphone array node |

The ReSpeaker provides two functions:

- Sound source direction (0-360°) — used by face_me command to orient the robot

- Microphone input for Vosk STT and wake word detection

## 10.2 Audio Playback

- Enter the full path to a WAV file on the robot (e.g. /home/robin/robin_ws/audio_files/beep.wav)

- Tap Play Audio — robot plays the file through its USB speaker

- Tap List Audio Files — shows all available WAV files in audio_files/ directory

💡 TIP Common audio files are in /home/robin/robin_ws/audio_files/. Important ones include: blip.wav (wake word confirmation), beginningtracking.wav (follow mode start), trackingstopped.wav (follow mode end).

# 11. Troubleshooting

| Problem | Likely Cause | Fix |
| --- | --- | --- |
| App shows no robot data | API server not running | SSH to robot, run: bash run.sh |
| Voice commands not detected | Wake listener not active | Open Voice Commands panel → tap BEGIN |
| Camera feed error | nvargus-daemon crashed | SSH: sudo systemctl restart nvargus-daemon |
| Scan returns 'Scan failed' | Zenon API unreachable | Check internet connection on robot |
| Robot doesn't recognize faces | DB empty or node not active | Re-enroll faces using face_enroll.py |
| Motion doesn't execute | Robot in wrong mode | Ensure mode is 'idle' before running motions |
| YOLO tracking stuck | node_manager didn't stop nodes | SSH: pkill -9 -f follower_ctrl |
| 'z bot' not heard | Gain too low or mic off | Check ReSpeaker is ON in Audio panel |

# 12. Quick Reference Card

## All Voice Commands at a Glance

| Category | Say After 'z bot' | Result |
| --- | --- | --- |
| Movement | go forward / go back | Robot drives |
| Movement | turn left / turn right | Robot turns |
| Movement | spin / stop | Spin or halt |
| Movement | face me | Faces your voice direction |
| Gesture | wave / dance / charge | Performs gesture |
| Vision | scan now | Describes current view |
| Vision | read text | Reads visible text (OCR) |
| Vision | scan id | Extracts ID card fields |
| Vision | describe scene | 2-sentence scene detail |
| Vision | scan mode on/off | Toggle continuous scan |
| Identity | checkface | Identifies person in view |
| Follow | follow me | Robot follows you |
| Follow | stop following | Robot stops following |
| System | deactivate | Robot sleeps |
| Any | [question] | Answered by LLM (≤30 words) |