ROBIN ROBOT

TECHNICAL DOCUMENTATION

Complete Engineering Reference

Platform: NVIDIA Jetson Orin NX  |  OS: Ubuntu 22.04  |  ROS2: Humble  |  Version: 1.0

# 1. Executive Summary

Robin is an interactive social robot built on ROS2 Humble running on an NVIDIA Jetson Orin NX. It combines real-time face recognition, voice interaction, vision-based scanning, autonomous object following, and expressive UI rendering into a unified system. Robin is designed for human-robot interaction in indoor environments — greeting known visitors, answering spoken questions, scanning documents/IDs, and physically responding to voice commands.

| Platform | NVIDIA Jetson Orin NX / Nano |
| --- | --- |
| OS | Ubuntu 22.04 (JetPack 36.4.7) |
| Middleware | ROS2 Humble |
| Motors | Dynamixel (joints) + DDSM400 (wheels) |
| Microphone | SEEED ReSpeaker 4-Mic Array (UAC 1.0) |
| Camera | CSI IMX sensor via GStreamer (nvarguscamerasrc) |
| Wake Word | OpenWakeWord — 'z bot' / 'Zee Bot' |
| STT | Vosk (local, offline) — /opt/models/vosk-en |
| LLM | Zenon Robotics Aggregator API (AIHIVE-LLMDSTXT) |
| TTS | Piper (local CUDA) — en_US-lessac-medium.onnx |
| Vision API | Zenon Robotics API (AIHIVE-LLMFILTXT) |
| Object Detection | YOLOv8 (Ultralytics, TensorRT engine) |
| Face Recognition | face_recognition lib (dlib HOG + CNN) |
| Face DB | SQLite — /home/robin/robin_ws/face_profiles.db |
| Flutter UI | Cross-platform control app |
| Web UI | FastAPI HTTP server + WebSocket |

# 2. System Overview

## 2.1 Main Capabilities

- Wake word detection ('z bot') → voice command processing → action execution

- Face recognition: enroll, identify, greet known visitors

- Vision scanning: describe scenes, read text (OCR), extract ID card fields

- Object detection (YOLOv8) and person following

- Motion control: Dynamixel joint servos + DDSM400 wheel motors

- Expressive robot face rendered via PyQt5 SVG or HTML (browser-based)

- LLM-powered Q&A via Zenon Robotics cloud API

- Flutter mobile app + web browser control interface

## 2.2 Major Subsystems

| Subsystem | Description |
| --- | --- |
| Voice Pipeline | Wake word → Vosk STT → intent router → LLM or action |
| Face Recognition | CNN/HOG detection, SQLite DB, consent-based enrollment, greeting |
| Vision / Scan Mode | GStreamer camera → JPEG → Zenon Vision API → TTS output |
| YOLO Detection | YOLOv8 TensorRT engine → object text + JSON → follower |
| Object Following | YoloDetector + ObjectTracker + FollowerCtrl nodes |
| Motion Control | Dynamixel joints (DXL wrapper) + DDSM400 wheels (wheel wrapper) |
| Robot Face / UI | PyQt5 SVG face node or HTML browser face, expression via ROS topic |
| Audio I/O | ReSpeaker 4-mic array (input), USB audio device (output), Piper TTS |
| API / Web Server | FastAPI HTTP (port 8000) + WebSocket, Flutter app backend |
| Node Manager | ROS2 service — starts/stops yolo and follow_obj on demand |
| Launch / Orchestration | run.sh + ros2 launch → all nodes start in correct order |

# 3. Top-Level Architecture

## 3.1 Node Graph Overview

The following table shows every major node, its role, and its primary communication links:

| Node | Type | Publishes | Subscribes / Calls |
| --- | --- | --- | --- |
| camera_publisher_node | Lifecycle | /camera/image/raw, /camera/camera_info | — |
| face_recognition_node | Lifecycle | /face/detection, /face/last_known, /face/debug_image | /camera/image/raw |
| wake_listener_node | Lifecycle | /robot/mode, /robot_face/expression, /robot/run_scenario, /scan/mode, /scan/trigger | /face/detection, /face/last_known, /robot/voice_inject, /yolo/detection_text, /microphone_array/direction |
| scan_mode_node | Lifecycle | /scan/result | /camera/image/raw, /scan/mode, /scan/trigger, /face/detection, /face/last_known |
| yolo_detector | Lifecycle | /yolo/detection_text, /yolo/detection_raw, /yolo/debug_image | /camera/image/raw |
| object_tracker_node | Lifecycle | /tracker/target | /yolo/detection_raw |
| follower_ctrl_node | Lifecycle | /ddsm400/cmd_speed | /tracker/target, /robot/mode |
| microphone_array_publisher_node | Lifecycle | /microphone_array/direction, /microphone_array/raw | — |
| face_node | Regular | /robot_face/expression (sub) | — |
| node_manager | Regular | /node_manager/status, /robot/mode | /robot/mode, /node_manager/shutdown_request |
| api_server (FastAPI) | HTTP | HTTP REST + WebSocket | Internal ROS + ai_chatbot |

# 4. Full System Data Flow

## 4.1 Voice Command Flow (Primary)

Voice Command Pipeline

1. Microphone → ReSpeaker 4-mic array captures audio (6-ch, 16kHz) 2. Wake Word → OpenWakeWord model (Zee_Bot.onnx) detects 'z bot' phrase 3. Beep + Record → aplay blip.wav, switch state to RECORDING 4. STT → Vosk KaldiRecognizer (local, offline) transcribes speech 5. Intent Router → deterministic regex matching → action intent or 'query' 6A. Action intent → publish to /robot/run_scenario → motion execution 6B. Query intent → Zenon LLM API (async poll) → clean response text 7. TTS → Piper local (CUDA) → WAV file → aplay -q → speaker output

## 4.2 Face Recognition Flow

Face Recognition Pipeline

1. camera_publisher_node publishes /camera/image/raw at 21 Hz (BEST_EFFORT QoS) 2. face_recognition_node receives frame → HOG pre-scan (0.5x scale) → skip if no faces 3. CNN detection (full-res) → 128-float embeddings via face_recognition library 4. DB match (SQLite) → if distance < 0.60: KNOWN → greet (10min cooldown) 5. If UNKNOWN: ask consent → YES: capture name via Vosk → save to DB 6. Publishes: /face/detection (every detection) + /face/last_known (every known face) 7. scan_mode_node and wake_listener_node subscribe to use name in responses

## 4.3 Vision Scan Flow

Scan Mode Pipeline

1. Voice trigger: 'z bot, scan now' → wake_listener → /scan/trigger 'auto' 2. scan_mode_node checks /face/detection for known person (< 5s fresh) 3. Build face-aware prompt: 'Robin is in this image. Describe what you see.' 4. Capture frame from /camera/image/raw → encode JPEG 5. POST to Zenon API /CreateRequest (AIHIVE-LLMFILTXT) with image + prompt 6. Poll GET /GetRequestByRequestId until status=2 (Done) — typically 4-8s 7. Extract result → strip markdown → inject known name → Piper TTS → speak

# 5. Node-by-Node Documentation

## 5.1 camera_publisher_node.py

| Purpose | Publish CSI camera frames to /camera/image/raw for all vision nodes |
| --- | --- |
| Type | ROS2 LifecycleNode |
| Location | /home/robin/robin_ws/src/camera_pkg/camera_pkg/camera_publisher_node.py |
| Hardware | IMX CSI ribbon cable sensor → nvarguscamerasrc GStreamer pipeline |
| Output | /camera/image/raw (sensor_msgs/Image, 960x540, BGR, 21Hz, BEST_EFFORT) |
| Output | /camera/camera_info (sensor_msgs/CameraInfo, calibration data) |
| QoS | BEST_EFFORT + VOLATILE + depth=1 (drop old frames, minimize latency) |
| Resolution | 3280x2464 native → nvvidconv → 960x540 BGR output |
| Rate | 21 Hz (matches sensor framerate) |

### Internal Logic

- on_configure: opens GStreamer pipeline, creates publishers, creates timer (cancelled)

- on_activate: starts timer → timer_callback runs at 21Hz

- timer_callback: cap.read() → cv2_to_imgmsg → publish Image + CameraInfo

- Failure recovery: if consecutive read failures ≥ 15, marks cap=None for reconnect

- Reconnect: releases pipeline, sleeps 1s, reopens GStreamer, tests with one frame

- Graceful shutdown: try/except around all destroy calls to avoid rclpy double-shutdown

Failure Modes

- 'Unable to read from camera' — GStreamer pipeline dropped; auto-reconnect triggers - 'Failed to create CaptureSession' — nvargus-daemon crashed; fix: sudo systemctl restart nvargus-daemon - InsufficientMemory — GPU OOM from multiple pipeline opens; fixed by single-process ownership - rclpy shutdown already called — fixed with try/except in main() finally block

## 5.2 face_recognition_node.py

| Purpose | Identify known faces from camera stream, greet them, enroll unknowns |
| --- | --- |
| Type | ROS2 LifecycleNode |
| Subscribes | /camera/image/raw (BEST_EFFORT QoS) |
| Publishes | /face/detection (String/JSON — every detection) |
| Publishes | /face/last_known (String/JSON — every recognized face, persistent) |
| Publishes | /face/debug_image (annotated Image) |
| Parameters | match_threshold=0.60, detect_interval=1.0s, greeting_cooldown=600s |
| DB | /home/robin/robin_ws/face_profiles.db (SQLite, 128-float embeddings) |
| Models | face_recognition (HOG for pre-scan, CNN for embedding) |
| Audio | Piper TTS for greetings + consent + name capture; Vosk for name input |

### State Machine

| State | Description |
| --- | --- |
| IDLE | Normal scanning state — processes every frame |
| GREETING | Known face found — speaking greeting, 10min cooldown tracking |
| ASKING_CONSENT | Unknown face — asking 'Would you like me to save your profile?' |
| LISTENING_CONSENT | Vosk listening for yes/no response |
| ASKING_NAME | Consent given — asking for person's name |
| LISTENING_NAME | Vosk listening to capture name |
| SAVING | Saving name + embeddings to SQLite DB |

### Detection Pipeline

- Stage 1: HOG detection at 0.5x scale (fast CPU) — if no faces found, return silently

- Stage 2: CNN detection at full resolution (number_of_times_to_upsample=1, GPU)

- Stage 3: face_recognition.face_encodings → 128-float vector per face

- Stage 4: L2 distance compare against all DB embeddings → if min_dist < 0.60: KNOWN

- Stage 5: Greeting cooldown check (600s per person) → speak greeting via Piper TTS

- Unknown: 5-minute ignore timer per face position to avoid repeated prompting

## 5.3 wake_listener_node.py

| Purpose | Primary voice interaction hub — wake word, STT, intent routing, dispatch |
| --- | --- |
| Type | ROS2 LifecycleNode (MultiThreadedExecutor) |
| Location | /home/robin/robin_ws/src/voice_pkg/voice_pkg/wake_listener_node.py |
| Wake Word | OpenWakeWord — /home/robin/robin_ws/audio_files/wakeword/Zee_Bot.onnx |
| STT | Vosk KaldiRecognizer — /opt/models/vosk-en (offline) |
| TTS | Piper — /opt/models/piper/en_US-lessac-medium.onnx (CUDA) |
| Microphone | sounddevice RawInputStream, 6-channel, 16kHz, channel 0 extracted |
| Publishes | /robot/mode, /robot_face/expression, /robot/run_scenario |
| Publishes | /scan/mode, /scan/trigger |
| Subscribes | /face/detection, /face/last_known, /robot/voice_inject |
| Subscribes | /yolo/detection_text, /microphone_array/direction |

### FSM States

- IDLE: OWW model processes 80ms chunks at 0.5 threshold — when detected → RECORDING

- RECORDING: Vosk streams live audio, VAD tracks silence → after 0.8s silence → EXECUTING

- EXECUTING: intent_router classifies → dispatch action or LLM query

- TRACKING: After 'follow me' — Vosk grammar-constrained, listens for 'stop following'

### Pre-Router Intercepts (never reach LLM)

- checkface / identify / detect me → _dispatch_checkface() → local DB → TTS

- scan_mode_on/off → publish to /scan/mode

- scan_now / read_text / scan_id / describe_scene → publish to /scan/trigger

- /robot/voice_inject topic → simulate voice command without speaking (testing)

## 5.4 scan_mode_node.py

| Purpose | Vision scanning — describe scenes, OCR, ID extraction via Zenon Vision API |
| --- | --- |
| Type | ROS2 LifecycleNode |
| API | Zenon Robotics AIHIVE-LLMFILTXT — POST image + prompt, poll for result |
| Subscribes | /camera/image/raw, /scan/mode, /scan/trigger |
| Subscribes | /face/detection (< 5s fresh), /face/last_known (< 10s fresh) |
| Publishes | /scan/result (String/JSON) |
| Scan Modes | auto (describe), ocr (read text), id (extract fields), scene, objects |
| Face Logic | Known face name injected into prompt AND post-processed into result text |
| TTS | Piper local (CUDA) → aplay |

### Face-Aware Prompt Logic

- Layer 1: /face/detection cache — if face seen < 5s ago: use that name in prompt

- Layer 2: /face/last_known — if timestamp < 10s old: use that name

- Layer 3: If no fresh face data → generic 'describe what you see' prompt

- Post-process: inject_name() replaces 'a person/the man/someone' with known name

- Result cleaning: strip **bold**, ### headers, \n escape, 'not found' lines (ID mode)

## 5.5 yolo_detector.py

| Purpose | Real-time object detection using YOLOv8 TensorRT engine |
| --- | --- |
| Type | ROS2 LifecycleNode |
| Model | yolov8s.engine (TensorRT, stored in workspace) |
| Subscribes | /camera/image/raw (BEST_EFFORT QoS) |
| Publishes | /yolo/detection_text (String — human-readable 'I see a person and a chair') |
| Publishes | /yolo/detection_raw (String/JSON — bbox data for tracker) |
| Publishes | /yolo/debug_image (annotated Image) |
| Parameters | confidence_threshold=0.5, detection_interval=0.1s, max_detections=5 |
| Lifecycle | Started on demand by node_manager when 'follow me' or 'what do you see' |

## 5.6 follower_ctrl_node.py

| Purpose | PID-like control loop for person/object following using wheel motors |
| --- | --- |
| Type | ROS2 LifecycleNode |
| Subscribes | /tracker/target (Vector3 — normalized x/y/size error) |
| Subscribes | /robot/mode (String — pauses if mode != tracking) |
| Publishes | /ddsm400/cmd_speed (DDSM400CmdSpeed — left/right RPM commands) |
| Control | Direct velocity mode: deadband=0.07, tracking_speed=10 RPM, lost_timeout=400ms |
| Smoothing | Low-pass filter on error signal (alpha=0.35) |
| Safety | Lost timeout → IDLE state, motor stop; mode check prevents spurious motion |

## 5.7 node_manager.py

| Purpose | Central lifecycle orchestrator — starts/stops yolo and follow_obj on demand |
| --- | --- |
| Type | ROS2 Node (MultiThreadedExecutor) |
| Service | /node_manager/control (NodeControl.srv) — start/stop nodes |
| Publishes | /node_manager/status (NodeStatus.msg), /robot/mode (String) |
| Subscribes | /robot/mode, /node_manager/shutdown_request |
| Managed | yolo (yolo_detector lifecycle), follow_obj (object_tracker + follower_ctrl) |
| Status | Publishes status for all managed nodes every 2 seconds |
| Note | Voice nodes (voice, ai_listen) are deprecated — wake_listener handles all |

## 5.8 microphone_array_publisher_node.py

| Purpose | Read SEEED ReSpeaker 4-mic array via USB, publish audio direction |
| --- | --- |
| Type | ROS2 LifecycleNode |
| Hardware | SEEED ReSpeaker 4-Mic Array (UAC 1.0) via pyaudio + usb.core |
| Publishes | /microphone_array/direction (Float32 — sound source angle 0-360°) |
| Publishes | /microphone_array/raw (Float32MultiArray — raw mic data) |
| DSP | Echo cancellation, noise suppression, VAD, AGC — via USB device registers |
| Consumer | wake_listener_node uses direction for face_me directional turning |

## 5.9 face_node.py

| Purpose | Render animated robot face using PyQt5 SVG — emotional expressions |
| --- | --- |
| Type | ROS2 Node + PyQt5 application |
| Subscribes | /robot_face/expression (String — 'default','listening','thinking','speaking','error') |
| Display | Full-screen SVG rendering, X11 display :0 |
| Expressions | default, listening, thinking, speaking, error — triggers animation |
| Alternatives | face.html, ultimate_robot_face.html — browser-based faces via roslib.js |
| Note | Requires DISPLAY=:0 and XAUTHORITY set in run.sh for boot launch |

## 5.10 ai_chatbot.py

| Purpose | AI pipeline module — STT, LLM, TTS via Zenon Aggregator or local services |
| --- | --- |
| Type | Python module (imported by wake_listener_node via api_server) |
| Modes | aggregator (cloud) or direct (local microservices) — via AI_MODE env var |
| STT | Zenon AIHIVE-WAVTXT or local — speech WAV file → transcribed text |
| LLM | Zenon AIHIVE-LLMDSTXT — text prompt → text response (≤30 words) |
| TTS | Zenon AIHIVE-TXTWAV — text → WAV URL (legacy) or Piper local |
| Polling | Async aiohttp poll loop (0.5s interval, 60s timeout) for all API calls |
| Note | wake_listener_node uses Vosk (local STT) and Piper (local TTS) directly — ai_chatbot.llm_query() used only for LLM step |

## 5.11 intent_router.py

| Purpose | Deterministic voice command classifier — no LLM for intent classification |
| --- | --- |
| Type | Python module (imported by wake_listener_node) |
| Input | Raw transcribed text string from Vosk |
| Output | Dict: {function: {name: intent, arguments: {}}} |
| Intents | face_me, turn_left/right, forward, backward, stop, spin, wave, charge, dance |
| Intents | what_do_you_see, who_am_i, follow_me, stop_following, deactivate |
| Intents | scan_mode_on/off, scan_now, read_text, scan_id, describe_scene, detect_objects |
| Fallback | question heuristic → query (LLM); ambiguous single word → dont_understand |
| Logic | strip wake word → normalize → split first clause → regex match with priority |

# 6. Hardware Interface Layer

## 6.1 Motor Controllers

| Controller | Interface | Notes |
| --- | --- | --- |
| Dynamixel | dxl_wrapper.py | Joint servos (head, arms) — trajectory + position commands via DXL lifecycle node |
| DDSM400 | ddsm400.py / wheel_wrapper.py | Wheel motors — /ddsm400/cmd_speed topic, RPM control, serial UART |

## 6.2 Sensor Hardware

| Sensor | Driver/Node | Notes |
| --- | --- | --- |
| CSI Camera | camera_publisher_node | IMX sensor, GStreamer nvarguscamerasrc, 21Hz, 960x540 |
| ReSpeaker 4-Mic | microphone_array_publisher_node | UAC 1.0 USB, 6-channel, 16kHz, pyaudio + usb.core |
| USB Speaker | aplay (subprocess) | USB audio output for TTS and sound effects |

# 7. Startup & Launch Sequence

## 7.1 run.sh Sequence

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Audio routing (pactl) | Set USB speaker as default sink; ReSpeaker as source; volume 30% |
| 2 | ROS environment | source /opt/ros/humble/setup.bash + workspace install/setup.bash |
| 3 | DISPLAY setup | Auto-detect DISPLAY=:0, XAUTHORITY — needed for face_node PyQt5 |
| 4 | Kill stale processes | pkill all camera, face, wake_listener, scan nodes + ros2 launch |
| 5 | ROS daemon reset | ros2 daemon stop/start — prevents rclpy.ok() faults |
| 6 | Start camera_publisher_node | python3 camera_publisher_node.py → sleep 6 → lifecycle configure → activate |
| 7 | ros2 launch robot_interface | Launches main stack: wake_listener, face_node, node_manager, api_server, etc. |
| 8 | face_recognition_node (delay 10s) | Delayed start: sleep 10 → python3 face_recognition_node.py → lifecycle configure → activate |
| 9 | scan_mode_node (delay 15s) | Delayed start: sleep 15 → python3 scan_mode_node.py → lifecycle configure → activate |

# 8. Draw.io Flowchart Specifications

## 8.1 Master System Architecture Diagram

Recreate this in draw.io using Grouped sections (swim lanes or container blocks):

HARDWARE LAYER (bottom)

- Block: CSI Camera → connects UP to camera_publisher_node - Block: ReSpeaker 4-Mic → connects UP to microphone_array_publisher_node - Block: USB Speaker → receives from Piper TTS (aplay) - Block: Dynamixel Servos → receives from dxl_wrapper - Block: DDSM400 Wheels → receives from follower_ctrl_node via /ddsm400/cmd_speed

DRIVER / PUBLISHER LAYER

- Block: camera_publisher_node → publishes /camera/image/raw → connects to face_recognition, yolo_detector, scan_mode_node - Block: microphone_array_publisher_node → publishes /microphone_array/direction → wake_listener_node

PERCEPTION LAYER

- Block: face_recognition_node → sub /camera/image/raw → pub /face/detection + /face/last_known - Block: yolo_detector → sub /camera/image/raw → pub /yolo/detection_text + /yolo/detection_raw - Block: object_tracker_node → sub /yolo/detection_raw → pub /tracker/target

DECISION LAYER

- Block: wake_listener_node (CENTRAL) → sub all perception topics → routes via intent_router - Block: intent_router (module) → deterministic → returns intent dict - Block: ai_chatbot.llm_query() → calls Zenon LLM API → returns text - Block: scan_mode_node → calls Zenon Vision API → returns result

CONTROL / OUTPUT LAYER

- Block: /robot/run_scenario → motion execution (scenarios in motion_library) - Block: follower_ctrl_node → sub /tracker/target → pub /ddsm400/cmd_speed - Block: face_node → sub /robot_face/expression → render PyQt5 SVG face - Block: Piper TTS → subprocess → aplay → USB speaker

## 8.2 Voice Command Flow Diagram

Blocks and arrows for draw.io:

| Block Name | Type | Arrow To | Arrow Label |
| --- | --- | --- | --- |
| Microphone (ReSpeaker) | Input | audio_loop (OWW) | 6-ch PCM 16kHz |
| audio_loop (OWW) | Process | Handle Wake Detection | score > 0.5 |
| Handle Wake Detection | Process | Piper TTS (blip) | play beep |
| Handle Wake Detection | Process | RECORDING state | state change |
| RECORDING state (Vosk) | Process | finish_recording() | silence 0.8s or 8s max |
| finish_recording() | Process | intent_router.route() | transcription text |
| intent_router.route() | Decision | _dispatch_action() | action intent |
| intent_router.route() | Decision | _dispatch_query() | query intent |
| intent_router.route() | Decision | _dispatch_scan() | scan intent |
| intent_router.route() | Decision | _dispatch_checkface() | checkface intent |
| _dispatch_action() | Output | /robot/run_scenario | scenario name string |
| _dispatch_query() | Process | ai_chatbot.llm_query() | text prompt |
| ai_chatbot.llm_query() | External | Piper TTS → Speaker | response text |

# 9. Dependencies & Interfaces

## 9.1 External APIs

| Service | Product Code | Used For |
| --- | --- | --- |
| Zenon Robotics Aggregator | AIHIVE-LLMDSTXT | LLM question answering (≤30 words) |
| Zenon Robotics Aggregator | AIHIVE-LLMFILTXT | Vision analysis (scan mode — image + prompt) |
| Zenon Robotics Aggregator | AIHIVE-WAVTXT | Cloud STT (legacy, Vosk used instead) |
| Zenon Robotics Aggregator | AIHIVE-TXTWAV | Cloud TTS (legacy, Piper used instead) |

## 9.2 Python Dependencies

| Package | Used By |
| --- | --- |
| face_recognition (dlib) | face_recognition_node — HOG/CNN detection + 128-float embeddings |
| opencv-python (cv2) | camera_publisher, face_recognition, yolo_detector, scan_mode |
| openwakeword | wake_listener_node — Zee_Bot.onnx wake word detection |
| vosk | wake_listener_node + face_recognition_node — offline STT |
| sounddevice | wake_listener_node — 6-channel microphone capture |
| aiohttp | ai_chatbot, scan_mode_node — async HTTP API calls |
| ultralytics (YOLO) | yolo_detector — YOLOv8 inference |
| PyQt5 | face_node — SVG robot face rendering |
| webrtcvad | wake_listener_node — voice activity detection for silence detection |
| fastapi + uvicorn | updated_api_server — HTTP REST + WebSocket server |
| pyaudio + usb.core | microphone_array_publisher_node — ReSpeaker control |

## 9.3 Key ROS Topics Reference

| Topic | Msg Type | Purpose |
| --- | --- | --- |
| /camera/image/raw | sensor_msgs/Image | Main camera feed — BEST_EFFORT QoS |
| /face/detection | std_msgs/String (JSON) | All face detections — known + unknown |
| /face/last_known | std_msgs/String (JSON) | Last recognized person — persistent |
| /robot/mode | std_msgs/String | System state: idle, listening, tracking, etc. |
| /robot/run_scenario | std_msgs/String | Motion command: turn_left, wave, dance, etc. |
| /robot_face/expression | std_msgs/String | Face expression: default, listening, speaking |
| /robot/voice_inject | std_msgs/String | Test: inject voice commands without speaking |
| /scan/mode | std_msgs/String | Scan mode control: auto, off |
| /scan/trigger | std_msgs/String | Trigger one-shot scan: auto, ocr, id, scene |
| /scan/result | std_msgs/String (JSON) | Scan results published for logging/UI |
| /yolo/detection_text | std_msgs/String | Human-readable YOLO output for TTS |
| /yolo/detection_raw | std_msgs/String (JSON) | Raw bbox data for object_tracker |
| /tracker/target | geometry_msgs/Vector3 | Normalized tracking error for follower |
| /ddsm400/cmd_speed | DDSM400CmdSpeed | Wheel motor RPM commands |
| /microphone_array/direction | std_msgs/Float32 | Sound direction 0-360° from ReSpeaker |
| /node_manager/control | NodeControl.srv | Start/stop managed nodes |

# 10. Failure Points & Troubleshooting

| Symptom | Cause | Node | Fix |
| --- | --- | --- | --- |
| Camera publishes nothing | nvargus-daemon crashed | camera_publisher | sudo systemctl restart nvargus-daemon |
| InsufficientMemory errors | Multiple GStreamer pipeline opens | camera_publisher | Ensure only one process opens CSI camera |
| 'Unable to read from camera' loop | GStreamer pipeline dropped | camera_publisher | Auto-reconnect handles it; 15 failures → reconnect |
| Face recognition: DB 0 enrolled | Wrong working directory / path | face_recognition | Pass -p db_path:=/home/robin/robin_ws/face_profiles.db |
| Wake word not detected | OWW threshold too high or gain too low | wake_listener | OWW_THRESHOLD=0.5, OWW_GAIN=2.0 — check mic levels |
| rclpy shutdown already called | Double-shutdown in camera node | camera_publisher | Fixed with try/except in main() finally block |
| Scan mode: 'No request ID' | Zenon API returns {value:} not {id:} | scan_mode_node | Use data.get('value') or data.get('id') |
| Voice inject: 'Waiting for subscription' | Old wake_listener version running | wake_listener | pkill -9 -f wake_listener_node && restart |
| Context.init() called twice | rclpy.init() called in both grab_frame and get_known_face_name | test_scan.py | Check rclpy.ok() before calling rclpy.init() |
| Wrong name in scan (always Robin) | DB last_seen used instead of live detection | scan_mode_node | Only use face data < 5s old; never query DB |

# 11. Quick Understanding Guide (New Engineer)

Welcome to the Robin robot codebase. Here is what you need to know first:

The 5 most important files to read first

1. run.sh — how the robot starts. Everything flows from here. 2. wake_listener_node.py — the brain. All voice commands flow through this. 3. intent_router.py — how voice commands become actions (no AI needed here). 4. face_recognition_node.py — face detection, enrollment, greeting logic. 5. scan_mode_node.py — vision scanning with Zenon cloud API.

The 3 key design decisions

1. Vosk for STT (offline, fast), Piper for TTS (local GPU). No latency from cloud audio. 2. Intent router is deterministic regex — intent classification never touches LLM. LLM only for open-ended queries. 3. CSI camera is exclusive — only camera_publisher_node can open it. Everyone else subscribes to /camera/image/raw.

# 12. Draw.io Build Checklist

- □ Master architecture diagram (hardware → driver → perception → decision → output)

- □ Voice command flow (mic → OWW → Vosk → intent_router → dispatch → TTS)

- □ Face recognition pipeline (camera → HOG → CNN → DB → greet/enroll)

- □ Scan mode flow (voice trigger → face context → Zenon API → clean → TTS)

- □ Object following flow (camera → YOLO → tracker → follower_ctrl → wheels)

- □ Startup / launch sequence (run.sh → camera → robot_interface launch → delayed nodes)

- □ FSM diagrams: wake_listener states (IDLE/RECORDING/EXECUTING/TRACKING)

- □ FSM diagrams: face_recognition states (IDLE/GREETING/ASKING_CONSENT/LISTENING_NAME/SAVING)

- □ Node manager control flow (service → start_node → lifecycle transitions)

- □ Full ROS topic graph (all nodes + topics + arrows + QoS profiles)

# 13. Missing Documentation

- Hardware wiring diagram: Dynamixel IDs, serial port assignments, power connections

- DDSM400 serial protocol: baud rate, packet format, motor IDs

- Camera calibration data: intrinsic matrix source, calibration procedure

- Full ros2 launch file contents: robot_interface.launch.py not in provided files

- Dynamixel joint map: which joint name maps to which DXL ID

- Network architecture: IP assignments, ports used, firewall rules

- Flutter app: API endpoints consumed, WebSocket protocol

- YOLO model location: where yolov8s.engine is stored, training details

- Audio file index: all .wav files in audio_files/ directory and their triggers

- Environment variables: complete list of AI_MODE, AI_API_BASE_URL, etc.

- Piper model config: whether .onnx.json config file is required alongside model

- Face enrollment procedure: step-by-step for adding new people to DB

- Recovery procedure: what to do when robot fails to boot / nodes crash

- Vosk model: which languages available, how to switch (AI_LANG env var)